Technical information about PC Card Loader

For technical support and advanced users

v1.65 - DD / COM One - 02/2000

1.	Troubleshooting	1

1.1	Macintosh Loader	1

1.2	Loader schematic	1

1.3	Miscellaneous	1

1.4	Software LOADER v1.xx under Windows 95	1

2.	PC Card LOADER releases	2

3.	Removing fully a Windows 95/98 modem	3

4.	Clearing PC Card “Plug and Play” keys	4

4.1	Do it only if	4

4.2	Start	4

5.	Windows PCMCIA Card Modem Loader : advanced help form	4

# 1.	Troubleshooting

## 1.1	Macintosh Loader

You can find Macintosh specific help in the "COM1MAC.DOC" file.

## 1.2	Loader schematic

LOADER.EXE	Graphic interface

FIRMWARE.INI	Firmwares files database.

CSAPI.DLL		Low level PCMCIA access, use CSINT.VXD for Windows 95/98, or CSINTNT.SYS for Windows NT 4.

## 1.3	Miscellaneous

Troubleshooting LOADER v3.xx

----------------------------

The Loader v3.xx and previous doesn't work with Windows NT. You must use another software : DOS CONFIG21.EXE

## Software LOADER v1.xx under Windows 95

To recognize v1.x : there's no version number displayed.

Q : I can't find the file "xxxxxxxx.BIN"

A : At the place "The firmware your are going to load is:". Click the button "Browse...", into the window "Open", click the button "Files of type:" and select "All files (*.*)".

Then follow the instructions from your paper manual.

Q: Error message "PCMCIA Card loader error, PCMCIA Driver: card memory is write protected."

A: In the Configuration Panel, double click on the PC CARD/PCMCIA icon.

select the modem card

click on the Stop button

click on OK

Click again on OK.

Do not eject the modem card, start the LOADER installation from the start.

Q: Error message "The firmware version of the selected file doesn't correspond to the PCMCIA card version"

French : "La version du firmware sélectionné ne correspond pas à la carte PCMCIA sélectionnée."

A: See the previous reply.

# 2.	PC Card LOADER releases

| 28/11/97 | 2.04 | Added : support for "GSM only" firmwares (Mobile Data PC Card). |
| --- | --- | --- |
| 29/01/98 | 3.00 | Added : support for MC220. Firmware modes: buttons changed (now only PSTN/ISDN or PSTN/ISDN/GSM). Error message when run under from WinNT4 : "This program cannot be run under Windows NT". |
| 20/03/98 | 3.02 | Bug removed. |
| 24/03/98 | 3.04 | A Problem appear 3/5 times to flash cards onto IBM TP 560, Windows 95. Added: Reset of the PC Card before flashing. |
| 10/04/98 | 3.05 | Beta release for Windows NT 4 |
| 17/04/98 | 4.00 | Added support of Windows NT 4. Fixed : HP OmniBook 3000. |
| 17/04/98 | 4.01 | Added : Support for more PCMCIA controllers. |
| 05/06/98 | 4.02 | Added : Reset PCMCIA controller before to flash. Added : Removes old MC220 INF keys from the registry to avoid to display "MC220 #1, MC220 #2" into the Loader window. |
| 01/10/98 | 4.03 | Added : LAN upgrade support. Fixed : problem bad cleaning of INF files if MC220 uninstalled. |
| 19/10/98 | 4.04 | Fixed : a WinNT 4 setting. Win95 : PC Card restart after flashing improved. |
| 27/10/98 | 4.05 | Added: Support for 4 Mb EPROM. |
| 28/10/98 | 4.06 | Fixed : Something for WinNT4 SystemSoft Card Wizzard v4.1. Error message was something as "No such device" (before the first window where you select the slot). (HP OB 800 CT with WinNT 4, TI 1131) Win95 : This version should remove the problem with "02 Micro 0Z6832 CardBus controller". Fixed: Hinote digital VP575, Win95 (Cirrus Logic 6832 PCMCIA / Card Bus controller). Sys 1.02 |
| 15/11/98 | 4.07 | OEM modifications. |
| 16/11/98 | 4.08 | Add : German language support. |
| 10/12/98 | 4.09 | Fix for PCMCIA controller : Intel 82365 under WinNT4 SP3 (ex: Siemens scenic mobile 510). Error message : "Time out has occured while waiting PCMCIA card to respond." Now display "Sys 1.03" |
| 27/05/99 | 4.09 (vxd 1.04) | Sharp PC-9080 under WinNT 4 : fix "PC freeze when clicking first Next button". |
| 31/05/99 | 4.10 | Add recognition of MC221 Discovery. Add support of MC222. |
| 01/10/99 | 4.11 | Modification for support of Windows 2000. |
| 01/11/99 | 4.12 | Add support of MC220 Liberty 4 Mb. |

# 3.	Removing fully a Windows 95/98 modem

Here's the best way to remove the old driver of a modem under Win95/98:

1) Eject the PC Card modem or switch it off.

2) Control pannel/modems/remove the modem driver (example: "COMOne MC220...").

3) Remove the INF file

3.1) For Win95 4.00.950 and 4.00.950a :

Go into C:\WINDOWS\INF\,

sort the files by date, and check which ones are used by the modem. Most of the times the files names are: "OEM?.INF". Delete these files or move them to another directory (usually 1 or 2).

Delete DRVDATA.BIN and DRVIDX.BIN

3.2) For Win95 4.00.950 B :

Go into C:\WINDOWS\INF\OTHERS\, and delete the "COMOne SA MC220.INF" file.

Go into C:\WINDOWS\INF\ then sort the files by type, delete DRVDATA.BIN and DRVIDX.BIN

3.3) For Win98 :

Go into C:\WINDOWS\INF\, and delete the "MDM220.INF" and "MDM220.PNF" files.

Go into C:\WINDOWS\INF\ then sort the files by type, delete DRVDATA.BIN and DRVIDX.BIN

Note: If you don't see the "INF" directory, go to Explorer / menu View /  Folder Options / Tab View, check "Hidden Files / Show all files".

4) Now the next time the system will detect a new modem, it will ask you for the INF driver file.

Note about DRVDATA.BIN and DRVIDX.BIN: These are the system drivers database. When the system find a new device, these files are automatically created by the system if they don't already exist.

# Clearing PC Card “Plug and Play” keys

This is the “ last chance ” way to uninstall a PC Card when Win95/98 do not detect it any more.

## 4.1	Do it only if

- You have removed the PC Card driver files from Win95/98 (previous chapter).

- When you insert the PC Card Win95/98 does not detect a new device and does not ask you a disk containing the driver.

## 4.2	Start

Eject the PC Card, then do the previous chapter process (“Removing fully a Windows 95/98 modem”), then come back here.

Run REGEDIT, go to HKEY_LOCAL_MACHINE\Enum\PCMCIA

If you see a “PCCARD” entry, this mean the user has installed the PC Card as a “Standard PCMCIA Card Modem”. Remove this key, close REGEDIT, the insert the PC Card and provide the disk 1 of the latest upgrade.

# 5.	Windows PCMCIA Card Modem Loader : advanced help form

Windows PCMCIA Card Modem Loader : advanced help

Document v1.33, 01/2000.

Best way to upgrade a PC Card under Win95/98

--------------------------------------------

1. Close all applications that may access to the PC Card (ex: HyperTerminal).

2. Remove the PC Card from its socket, wait 5 seconds, then insert it again.

3. Go into : Control panel / System / PC Card (PCMCIA), select the PC Card then click the button "Stop". Do not remove the PC Card, then run the Loader.

4. If you own a ISDN, GSM, or LAN adapter, unplug it from the card.

Here are the information the support will need

----------------------------------------------

- Computer FULL name : ...

- Operating system name and version (services packs installed?) : ...

- Was the OS installed by the manufacturer of the PC (OEM OS), or from a standard CD-Rom ? : ...

- Loader version (ex: "4.09 WIN (vxd 2.02)"): ...

- What's the error message or the last window? : ...

If the Loader stopped after starting to upgrade the PC Card, please give us the status message (ex: "Stopping PCMCIA card", "Loading firmware file", "Hardware setup", ...).

- PC Card product name : ....

- Is any PCMCIA manager installed? (ex: System Soft Card Wizard) : ...

Did you installed this software yourself, or the manufacturer did? : ...

- PCMCIA controller name : ...

Win95/98: Control panel / System / PCMCIA socket, then write down all the lines inside.

WinNT 4: Run the Loader 4.xx select the PC Card, then read the name of the controller at the bottom of the window.

Email this form to : support@com1.fr

Questions / Answers

-------------------

Q: error message : "Time out has occured while waiting PCMCIA card to respond."

French error message: "Erreur du chargeur de carte PCMCIA : La carte PCMCIA ne répond pas".

A: Unfortunately this version of the Loader isn't compatible with this computer. Try to get the latest Loader version or upgrade the card onto another PC.

Q: Under Win95/98 Is it normal that the name of the MC220 appears as "Standard PCMCIA Card Modem" ?

A: After the upgrade, Windows has detected a new modem and asked you the matching INF driver file. You've probably inserted the PSTN only disk 1, instead of disk 1 of the kit used. Windows has not found the right driver then proposed "Standard PCMCIA Card Modem" and you agreed.

The main problem is that this will not allow to display K56 Flex/V90 real connection speeds.

Solution: you must uninstall the current driver:

1. Eject the PC Card.

2. Control pannel/modems/remove the modem driver (example: "Standard PCMCIA Card Modem" and others related to this PC Card).

3. Insert the PC Card.

4. Provide the latest MDM220.INF file.

Q : error message "A system error was detected: The system cannot find the path specified".

A: If you run one time the Loader v4.xx under WinNT 4, one of the file's path is written into the registry. So if you move the Loader's files to another directory then run it again, you get the error message.

Solution:

- Run REGEDIT.EXE

- Go into: HKEY_LOCAL_MACHINE

- And delete this key: \\SYSTEM\CurentControlSet\Control\Services\Csintnt

Q : error message "... 65535"

French: "Erreur du chargeur de cartes PCMCIA. Code 65535.".

Reported with Loader v4.09 (from TECH0330) under Windows 98.

A: Quit Loader. Eject the PC Card, wait for 10 seconds then insert it, then run the Loader again. Perhaps you may have to restart Windows.