Drivers releases

For technical support and advanced users

V1.70 - DD / COM One - 02/2000

Introduction	1

MC218 drivers releases	1

MC218 ISDN Companion for Win95	1

MC218 ISDN Companion for Windows NT 4	4

MC218 ISDN driver v1.6 for Win95/98	4

MC218 ISDN driver v1.6 for WinNT 4	6

MC220 Platinium Card 56K	8

MC220 PSTN (default disk) TECH0021 (1 disk)	8

MC220 ISDN TECH0167 (6 disks)	8

MC220 LAN driver (4 disks)	10

MC220 ISDN KIT v1.8 TECHxxxx (7 disks)	11

MC220 GSM KIT	12

MC220 GSM KIT, TECH0182 (4 disks)	12

MC220 GSM KIT, TECH0225 (4 disks)	13

MC220 Platinium CD Rom revisions	14

TECH0260 (DISK0177)   03/11/98	14

TECH0264  13/11/98	14

TECH0265  17/11/98 (OEM 18/11/98)	14

Patch files released	15

TECH0330 (DISK0213)   13/03/99	16

Firmwares files included	17

Patch files released	20

TECH0359 29/09/99	21

Patch files released	22

MC222	22

TECH0390 13/12/99	22

# Introduction

This document concern drivers releases for MC218 and MC220 PC Cards.

Latest releases are described at the end of the document.

# MC218 drivers releases

## MC218 ISDN Companion for Win95

V1.5b FR/US 30/09/97

--------------------

Better results to receive calls.

CAPI 2.0 don't work with Win95 OSR2 (a file is missing).

Problem known at uninstallation: the uninstaller forget to clear some files.

Before to install the new driver, run RMSXFLS.EXE from DISK2 to clear to files forgotten.

V1.5c FR/US 15/10/97

--------------------

Internal :

- VXD's organisation modifications, in order to solve various hang up with PC Rider 128 (ATA, fast connection/deconnection cycles).

- Set to standard of some AT commands (RING, A/, ...).

Public :

- CAPI Installation : Into previous releases, CAPI installation under Windows 95 OSR2 don't work (VCAPID.VXD was deleted).

- Corrections into languages English and German (menus and configuration assistants).

- Corrections for parity function (HyperTerm can't change it into previous releases).

- Add function of command mode under HyperTerm (command +++).

- Junction (CD, DTR, ...)

- Auto answer mode: ATS0=0 now display RING's.

- Fix problems with Windows 95 Remote Access Server (RAS).

- Fix problems with Shiva Remote Connect (v5.01 for Windows 95). Into previous ISDN Companion releases, in case of connection error, the modem reset done by Shiva killed the SX128 modem.

- Remark: Shiva Remote Connect has a problem in TAPI mode. It doesn't dial the last digit of a phone number. Solution: add a dummy digit at the end of phone number (ex: to call 12 34 56 78 with Shiva and SX128 - WAN Miniport Adapter, you must call: 12 34 56 78 9).

- Shiva Remote Connect can't work into CAPI mode currently.

- Uninstallation: the uninstaller RMSXFLS.EXE clear all the files (except the modems installed into Control panel).

- Added: virtual modems "SX128 MODEM PPP", "SX128 MODEM V120", "SX128 MODEM X75", to avoid to change protocol into the SX128 panel control.

These modems automatically sent init strings to set the protocol.

To add one of these modems: go into Control Panel / Modems, then "Add", display list, constructor: "ComOne".

Constructor name (ComOne) will change for the next release.

V1.5c FR/US 15/12/97

--------------------

TECH0058 : D320 et D321 (FR).

TECH0059 : D322 et D323 (US).

Masters sans boot WinImage 4 donc sans fausse alerte virus de la part de NAV.

V1.5d US 05/12/97

-----------------

Add MSN reply function (multiple suscribes numbers). Example (123 for sub address, and 0556788382 for MSN), go SX128 control panel / Setup / SX128 Modem, into the "Local sub address" box, enter :

- "123" for a sub adress number.

- "#0556788382" for a MSN number.

07/07/99 Wrong: "#0556788382*123" for a MSN and a sub adress number.

You can only set a MSN OR a sub adress.

Analog devices (data, fax) don't support sub adresses.

Analog part of the PC Card do not support sub adresses.

MSN (English) = SDA (French)

Sub address = France Sous adresse

V1.5d FR/US 16/02/98

--------------------

Does not permit to receive PPP nor V120 calls.

V1.5e FR/US 09/04/98

--------------------

Add: MC220 support.

Fix Win95 Performance Monitor doesn't display any longuer xx % CPU.

SETUPSX.EXE renamed to TUPSX.EXE

Fix for SHIVA dialing.

Fix for PCMCIA Texas Instruments and Cirrus Logic.

Fix for script RAS (ex: Compuserve).

ATI3=1.5d

V1.6b FR/US 01/01/99

--------------------

Same correction as for MC220 ISDN KIT v1.8 TECHxxxx (7 disks)

Uninstallation Guide

--------------------

For all versions from v1.0 to v1.6b

Installation and uninstallation Guide

-------------------------------------

CAUTION ! This driver was made only for Windows 95/98. Do NOT install it onto Windows NT 4 nor other system.

The 2 files you have downloaded contain the files of 2 disks.

- Onto your hard disk, make 2 directories (ex : C:\TEMP1 et C:\TEMP2)

- Copy the first file into the first directory, and the second file into the second directory.

- Run the EXE files you have copied.  The 2 EXE will decompress and create a lot of other files.

Then delete the 2 EXE files from their directories, they are no longer usefull.

- Copy the files of the directories onto 2 empty disks.

- you must uninstall the previous ISDN Companion version : Follow the paper documentation "Uninstalling your ISDN Product". If you don't own this paper documentation, double click onto the SX128 icon (bottom right of the screen), then click onto button "SETUP", then click the button "Uninstall all".

- After the reloading of Windows 95, insert disk 2 in disk drive, then run "A:\RMSXFLS.EXE"

- Insert disk 1, then run "A:\SETUP.EXE" and follow your paper documentation.

- ATTENTION ! Ces pilotes sont destines exclusivement a Windows 95/98. Ne surtout PAS les installer sur Windows NT 4 ou autre système. 

Les deux fichiers que vous avez téléchargé contiennent l'équivalent du contenu de deux disquettes sous forme compactée (économie en taille Ko).

- Sur votre disque dur, créez 2 répertoires vides (ex : C:\TEMP1 et C:\TEMP2)

- Copiez le premier fichier dans le premier répertoire, et le deuxième fichier dans le deuxième répertoire.

- Exécutez les fichiers EXE que vous venez de copier. Ils vont se décompacter et générer de nombreux autres fichiers. Supprimez ensuite vos deux anciens fichiers EXE, ils ne sont plus utiles.

- Copiez le contenu des répertoires sur des disquettes vierges.

- Il est necessaire de supprimer la version precedente d'ISDN Companion : Suivez la documentation papier a la page "Desinstallation des logiciels".

- Apres le relancement de Windows 95, inserez la disquette 2 dans le lecteur, puis executez "A:\RMSXFLS.EXE"

- Inserez la disquette 1, et executez "A:\SETUP.EXE" en suivant la documentation papier.

## MC218 ISDN Companion for Windows NT 4

V1.10 25/02/98

--------------

Added : support MC218 firmwares Generic.

Known problem: if the system WinNT is run without the ISDN interface plugged to the PC Card, the software SX2CARR.EXE will take about 95% of CPU usage. The computer will be slower.

V1.20 20/03/98

--------------

Added : analogic modulation (data, fax, French Minitel).

Fixed: SX2CARR.EXE taking 95% of CPU.

Update: Avoid dialling problems with Shiva Remote Access 5.xx

DOC: Added Troubleshooting section.

V1.30 10/03/98

--------------

ISDN Support for MC220 only (not MC218).

V1.40 30/03/98

--------------

Support MC220, MC218, and lot of OEM PC cards.

Window "attention" before to install the driver.

Trident: "COM1 MC21x not found" corrected.

V1.60 22/05/98

--------------

Fixed : no more COM port moving. So no more error into the EventViewer.

Fixed : Error message "your COM1 MC218" is not installed (OEM request, sxisdn.ini).

V1.70 01/12/98 for MC220

--------------

Added: Support together ISDN + MC220 LAN.

New: Install Shield package.

## MC218 ISDN driver v1.6 for Win95/98

MC218 ISDN KIT v1.6, TECH0... (2 disks)

COM One Support technical note 28/09/98

Product description

-*-*-*-*-*-*-*-*-*-

- ISDN drivers for Win95/98 only.

Installation remarks

-*-*-*-*-*-*-*-*-*-*

Caution !

---------

This driver was made only for Windows 95/98.

Do NOT install it onto Windows NT 4 or any other system.

How to install the new version of "ISDN Companion" software US

--------------------------------------------------------------

- The 2 files you have downloaded contain the files of 2 disks.

- Onto your hard disk, make 2 directories (ex : C:\TEMP1 et C:\TEMP2)

Copy the first file into the first directory, and the second file into

the second directory.

Run the EXE files you have copied.

The 2 EXE will decompress and create a lot of other files.

Then delete the 2 EXE files from their directories, they are no longer

useful.

- Copy the files of the directories onto 2 empty disks.

- You must uninstall the previous ISDN Companion version :

Follow the paper documentation "Uninstalling your ISDN Product".

If you don't own this paper documentation, then double click onto

SX128 icon (bottom right of the screen), then

click onto button "SETUP", then click the button "Uninstall all".

- After the reloading of Windows 95, insert disk 2 in disk drive, then

run "A:\RMSXFLS.EXE"

- Insert disk 1, then run "A:\SETUP.EXE" and follow your paper

documentation.

Problems fixed

-*-*-*-*-*-*-*

X75

---

X75 answering mode.

New PCMCIA controllers supported:

--------------------------------

Texas Instruments PCI-1230 Cardbus (IBM TP760)

Texas Instruments PCI-1231 Cardbus

Texas Instruments PCI-1250 Cardbus (IBM TP560X)

Cirrus Logic 6832 (Compaq Armada, AST)

Problems known

-*-*-*-*-*-*-*

ATI3

----

Problem: ATI3 reply "1.5d" instead of "1.6"

Solution: No fix. Please get isdn driver version number from SX128 control panel.

Installing CAPI under Win98

---------------------------

Problem: Under Win98, CAPI can't be installed. Error message: "SX128 MAC driver for Capi Subsystem must have VCAPID installed".

German error message: "VCAPID muss für SX128 MAC Driver for CAPI-Subsystem installiert sein".

Solution: Get the driver release v1.8 or further, uninstall the current driver, then install the new one.

ISDN mode not supported for this PC

-----------------------------------

Texas Instruments Extensa 650

Virtual modems: COM Port not switched by 16/32bits function

-----------------------------------------------------------

Problem : When you change the SX128 Modem from 32Bit only to 16/32 Bit it only changes the Com-Port of the SX128 Modem, but it didn't change the Com-Ports of the SX128 Modem PPP, V120, X75.

Solution : None. You can't use the virtual modems with COM1-4.

Incoming call's : MSN's

-----------------------

Problem: Can't receive a call using a Multiple Subscriber's number.

Solution: Use an "SX128 MODEM" and remove the local sub address ("01") from the SX128 control panel.

This problem is corrected for Win95/98 in the driver v1.8

## MC218 ISDN driver v1.6 for WinNT 4

MC218 ISDN driver v1.6, TECH ? (1 disk + doc file)

COM One Support technical note 05/10/98

Product description

-*-*-*-*-*-*-*-*-*-

- ISDN driver for WinNT 4 v1.6

Installation remarks

-*-*-*-*-*-*-*-*-*-*

Read the doc file *before* to start !!

Troubleshooting

-*-*-*-*-*-*-*-

WinNT 4 : Event Viewer error message

------------------------------------

Problem: If you boot NT with the MC218 inserted but the ISDN interface not plugged, you'll get errors messages in the Event Viewer.

2 times this message: Description: The SX128 ISDN STACK service depends on the following nonexistent service: sx2api.

1 time this message: The SX128 STACK RELAY service depends on the SX128 ISDN STACK service which failed to start because of the following error: The dependency service does not exist or has been marked for deletion.

Note: ISDN should still be available without restarting NT.

Solution: None.

Problems fixed

-*-*-*-*-*-*-*

- No more COM port moving. So no more error into the EventViewer.

- Error message "Your COM1 MC218 is not installed" (OEM request).

- Added : MC220 support.

Problems known

-*-*-*-*-*-*-*

Windows function "Disconnect if idle"

-------------------------------------

Problem: Win95/98 No "Disconnect if idle" option. Not able to set "Disconnect call if idle for more than X minutes" property for any type of ISDN connection. The field is greyed out.

Solution: None yet. Planed for the future.

Incoming call's : MSN's

-----------------------

Problem: Can't receive a call using a Multiple Subscriber's number.

Solution: Use an "SX128 MODEM" and remove the local sub address ("01") from the SX128 control panel.

This problem is corrected for Win95/98 in the driver v1.8

Protocols missing

-----------------

Problem: Some protocols are missing since the Win95/98 driver version.

Solution: None. The only protocols supported under WinNT 4 are: PPP and ML PPP (no CAPI nor V120 nor X75, no access via a virtual COM port).

# MC220 Platinium Card 56K

## MC220 PSTN (default disk) TECH0021 (1 disk)

Error message : "Setup requires a different version of windows. Check to make sure that you are running Setup on the windows platform for which it is intented". ERROR 102

Solution : This disk can't install under Windows 3. Build a PSTN disk with the Diskmaster application from MC220 CD-Rom (TECH0255 or next)

## MC220 ISDN TECH0167 (6 disks)

MC220 ISDN KIT v1.7, TECH0167... (6 disks)

COM One Support technical note 05/10/98

Product description

-*-*-*-*-*-*-*-*-*-

- PSTN HTML documentation.

- ISDN HTML documentation for Win95 and NT4.

- ISDN driver for win95 v1.6 (renamed to v1.7), uninstallation moved

from "SX128/Control panel/uninstall all" to Win95 and

WinNT "Add/remove softwares".

- ISDN driver for WinNT v1.6 (renamed to v1.7).

- New INF file "COMOne Platinium Card 56K Caller ID".

Installation remarks

-*-*-*-*-*-*-*-*-*-*

- Warning when unzipping the files, disk 1 have a sub-directory (pkunzip -d).

- To avoid uninstallation problems:

If you have a computer with Win95 *and* WinNT4, choose a different

directory when installing the files to avoid uninstallation problems.

Default value: C:\Program Files\COMOne\COMOne Platinium Card

Example for WinNT4: C:\Program Files\COMNT4\COMOne Platinium Card

Example for Win95/98: C:\Program Files\COMW95\COMOne Platinium Card

- Under Windows NT 4, Microsoft Internet Explorer v2 will not be able

to open the documentation at the end of installation (empty window).

You must run the documentation from the Start button/Applications.

- We recommend to read the documentations with a HTML browser supporting the frames (for example: Microsoft Internet Explorer v3.xx, Netscape Navigator v3.xx)

Troubleshooting

-*-*-*-*-*-*-*-

Tick sound

----------

Problem: Onto some PC's with Win95 (ex: HP Omnibook 3000, Win95 OSR2), a little "tick" sound can be played by the PC speaker every 10 to 30 seconds.

The tick occurs only if the ISDN driver is installed and the ISDN adapter not plugged to the PC Card.

This problem don't appear with a MC218 but only with a MC220.

Solution: None.

WinNT 4 : Event Viewer error message

------------------------------------

Problem: If you boot NT with the MC220 inserted but the ISDN interface not plugged, you'll get errors messages in the Event Viewer.

2 times this message: Description: The SX128 ISDN STACK service depends on the following nonexistent service: sx2api.

1 time this message: The SX128 STACK RELAY service depends on the SX128 ISDN STACK service which failed to start because of the following error: The dependency service does not exist or has been marked for deletion.

Note: ISDN should still be available without restarting NT.

Solution: None.

Problems known

-*-*-*-*-*-*-*

HLP files and screens

---------------------

Problem: Various misspellings and mistakes.

Solution: Corrections done in the driver v1.8

Windows function "Disconnect if idle"

-------------------------------------

Problem: Win95/98 No "Disconnect if idle" option. Not able to set "Disconnect call if idle for more than X minutes" property for any type of ISDN connection. The field is greyed out.

Solution: None yet. Planed for the future.

Incoming call's : MSN's

-----------------------

Problem: Can't receive a call using a Multiple Subscriber's number.

Solution: Use an "SX128 MODEM" and remove the local sub address ("01") from the SX128 control panel.

This problem is corrected for Win95/98 in the driver v1.8

Win98 only : CAPI

-----------------

Problem: CAPI can't be installed. Error message: "SX128 MAC driver for Capi Subsystem must have VCAPID installed".

Solution: This problem is corrected in the driver v1.8 (note this problem doesn't appear with Win95).

Win95/98 only : Call back with TAPI lines

-----------------------------------------

Problem: Using the device "SX128i-LineXX", the ISDN does not get the incoming call back.

Solution: Work around, use the "SX128 MODEM" device to call and receive call back.

This problem is corrected in the driver v1.8.

ATI3

----

Problem: ATI3 reply "Version 1.5d" instead of "1.7"

Solution: Please get isdn driver version number from SX128 control panel. This is fixed in driver v1.8

Virtual modems : COM Ports not switched by 16/32 bits function

--------------------------------------------------------------

Problem: When you change the SX128 Modem from 32Bit only to 16/32 Bit it only changes the Com-Port of the SX128 Modem, but it didn't change the Com-Ports of the SX128 Modem's: "SX128 MODEM PPP", "SX128 MODEM V120", "SX128 MODEM X75".

Solution: You can't use the virtual modems with COM ports 1 to 4. You must select "SX128 Modem" then choose the protocol into the SX128 control panel.

This problem is corrected in the driver v1.8.

Problems known with some computers

-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*

Various brands

--------------

Problem: After using CAPI, when trying to shut down Win95/98 the system freeze and never display the last screen ("You can now switch off your computer").

Solution: This problem is corrected in the driver v1.8.

Dell Latitude CP

----------------

Problem: With Dell's Latitude CP the Windows 95 system hangs on shutdown if any cable is connected to the ISDN adapter on the MC220 card. If the ISDN adapter is disconnected before shutdown there is no problem.

Solution: A patch file is available from COM1.

This problem is corrected in the driver v1.8.

Toshiba 300 CDT

---------------

Problem: Hazardous problem when booting a Toshiba 300 CDT. The Windows 95 system freeze : only the background color of the desktop appear with the mouse waiting cursor.

Solution: A patch file is available from COM1.

This problem is corrected in the driver v1.8.

## MC220 LAN driver (4 disks)

MC220 LAN kit before 15/10/98 (files date)

COM One Support technical note 23/10/98

This version of the driver was only spread for evaluation.

Troubleshooting

-*-*-*-*-*-*-*-

IP adress

---------

If you have already an network card installed in your system you must give different IP addresses for each network card.

Problems known

-*-*-*-*-*-*-*

HTML Documentation

------------------

Only LAN documentation for Win95 available, and contain some errors.

We are correcting this.

Under Windows NT 4

------------------

Address, IRQ, and COM port are set by default (D000, 3, 2).

If you need, change these values to the same as the MC220 PC Card analogue.

Changing after installation:

Control panel / Network / Adapters / SX456ETH / Properties.

Get COM Port value from Control panel / Modems.

Get address value from Control panel / PC Card (PCMCIA) / MC220 / tab Controller / Memory range (ex: D000-xxxx). Get IRQ.

In further versions, the installation software automatically get these values.

Under Windows 98

----------------

LAN+Modem can't work on the same time (modem don't transfer anything after some minutes but the LAN keep working).

Fixed with new MC220 firmwares versions (ww.l54, v90.l53, k56.l54).

## MC220 ISDN KIT v1.8 TECHxxxx (7 disks)

MC220 ISDN KIT v1.8, TECHxxxx (7 disks) from CD-Rom Platinium3

COM One Support technical note 03/11/1998

Troubleshooting

-*-*-*-*-*-*-*-

Incoming call's : MSN's

-----------------------

Problem: Can't receive a call using a Multiple Subscriber's number.

Solution: Use an "SX128 MODEM" and remove the local sub address ("01") from the SX128 control panel.

By default we set sub address 01 to avoid the PC Card to answer any ISDN incoming call.

Problems known

-*-*-*-*-*-*-*

Port "Maximum speed" = 57600

----------------------------

Problem : The maximum speed displayed is 57600.

Explanation: The SX128 Modem is a virtual, don't pay attention to this setting, it is not important. Once connected the real ISDN speed will be displayed (64000 or 128000).

Problems fixed

-*-*-*-*-*-*-*

HLP files and screens

---------------------

Various misspellings and mistakes corrected.

Win98 only : CAPI

-----------------

Problem: CAPI can't be installed. Error message: "SX128 MAC driver for Capi Subsystem must have VCAPID installed".

Solution: Fixed with ISDN driver v1.8

ATI3

----

Problem: ATI3 reply "Version 1.5d" instead of "1.7"

Solution: Fixed with ISDN driver v1.8 (ATI3 and AT*I reply "1.8"). We recommend to get isdn driver version number from SX128 control panel.

Virtual modems: COM Port not switched by 16/32bits function

-----------------------------------------------------------

Problem : When you change the SX128 Modem from 32Bit only to 16/32 Bit it only changes the Com-Port of the SX128 Modem, but it didn't change the Com-Ports of the SX128 Modem PPP, V120, X75.

Solution : Fixed with ISDN driver v1.8 release 2

Call back with TAPI

-------------------

Problem: Using the TAPI device "SX128i-LineXX", the ISDN does not get the incoming call back.

Solution: Fixed with ISDN driver v1.8. Remove the SX128 TAPI sub address if not used in the dial string.

Fixed problems known with some computers

-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*

Various brands

--------------

Problem: After using CAPI, when trying to shut down Win95/98 the system freeze and never display the last screen ("You can now switch off your computer").

Solution: Fixed with ISDN driver v1.8

Dell Latitude CP

----------------

Problem: With Dell's Latitude CP the Windows 95 system hangs on shutdown if any cable is connected to the ISDN adapter on the MC220 card. If the ISDN adapter is disconnected before shutdown there is no problem.

Solution: Fixed with ISDN driver v1.8

Toshiba 300 CDT

---------------

Problem: Hazardous problem when booting a Toshiba 300 CDT. The Windows 95 system freeze : only the background color of the desktop appear and the mouse waiting cursor.

Solution: Fixed with ISDN driver v1.8

# MC220 GSM KIT

## MC220 GSM KIT, TECH0182 (4 disks)

COM One Support technical note 04/06/98

Product description

-*-*-*-*-*-*-*-*-*-

- PSTN HTML documentation.

- GSM HTML documentation.

- Loader and MC220 firmwares.

- New INF file "COMOne Platinium Card 56K Caller ID".

Installation remarks

-*-*-*-*-*-*-*-*-*-*

- Warning when unzipping the files, disk 1 have a sub-directory (pkunzip -d).

- To avoid uninstallation problems:

If you have a computer with Win95 *and* WinNT4, choose a different

directory when installing the files to avoid uninstallation problems.

Default value: C:\Program Files\COMOne\COMOne Platinium Card

Example for WinNT4: C:\Program Files\COMNT4\COMOne Platinium Card

Example for Win95/98: C:\Program Files\COMW95\COMOne Platinium Card

- Under Windows NT 4, Microsoft Internet Explorer v2 will not be able

to open the documentation at the end of installation (empty window).

You must run the documentation from the Start button/Applications.

- We recommend to read the documentations with a HTML browser supporting

the frames (for example: Microsoft Internet Explorer v3.xx, Netscape

Navigator v3.xx)

## MC220 GSM KIT, TECH0225 (4 disks)

COM One Support technical note 01/09/98

Product description

-*-*-*-*-*-*-*-*-*-

- PSTN HTML documentation.

- GSM HTML documentation.

- Loader and MC220 firmwares.

- New INF file "COMOne Platinium Card 56K Caller ID".

Installation remarks

-*-*-*-*-*-*-*-*-*-*

- Warning when unzipping the files, disk 1 have a sub-directory (pkunzip -d).

- To avoid uninstallation problems:

If you have a computer with Win95 *and* WinNT4, choose a different

directory when installing the files to avoid uninstallation problems.

Default value: C:\Program Files\COMOne\COMOne Platinium Card

Example for WinNT4: C:\Program Files\COMNT4\COMOne Platinium Card

Example for Win95/98: C:\Program Files\COMW95\COMOne Platinium Card

- Under Windows NT 4, Microsoft Internet Explorer v2 will not be able

to open the documentation at the end of installation (empty window).

You must run the documentation from the Start button/Applications.

- We recommend to read the documentations with a HTML browser supporting

the frames (for example: Microsoft Internet Explorer v3.xx, Netscape

Navigator v3.xx)

Firmwares files included:

- Default

File1=COM1WW.L24,K56Flex+V90 PSTN / ISDN

- K56Flex PSTN / GSM / ISDN

COM1MOT.L28,MOTOROLA 8400 / 8700 / D460

COM1MOT.L28,MOTOROLA StarTac / SlimLite

COM1NOK.L28,NOKIA 8110 / 3110 / 3810

COM1NK2.L31,NOKIA 2110

COM1NK6.L01,NOKIA 6110 / 5110

COM1ERK.L32,ERICSSON GH388 / GF788

COM1SIE.L28,SONY CM-DX1000

COM1S10.L30,SONY CMD-X2000

COM1Z1.L28,SONY Z1/Z1PLUS

COM1SIE.L28,SIEMENS S4

COM1S10.L30,SIEMENS S10

- V90 PSTN / GSM / ISDN

COM1VMOT.L15,MOTOROLA 8400 / 8700 / D460

COM1VMOT.L15,MOTOROLA StarTac / SlimLite

COM1VNOK.L15,NOKIA 8110 / 3110 / 3810

COM1VNK2.L18,NOKIA 2110

COM1VNK6.L01,NOKIA 6110 / 5110

COM1VERK.L16,ERICSSON GH388 / GF788

COM1VSIE.L15,SONY CM-DX1000

COM1VS10.L17,SONY CMD-X2000

COM1VZ1.L15,SONY Z1/Z1PLUS

COM1VSIE.L15,SIEMENS S4

COM1VS10.L17,SIEMENS S10

# MC220 Platinium CD Rom revisions

Label of the CD Rom show the version, else look at the date of the latests files.

## TECH0260 (DISK0177)   03/11/98

Problem known : LAN/ISDN under WinNT4.

1. LAN driver corrected: on Sony VAIO LAN was not always seen.

2. ISDN 1.8 driver for Win95/98 changed :

Fixed :

-------

- Virtual modems : COM Port number not switched by 16/32bits function

Problem : When you change the SX128 Modem from 32Bit only to 16/32 Bit, it only changes the Com-Port of the SX128 Modem, but it didn't change the Com-Ports of the SX128 Modem PPP, V120, X75.

Solution : None. You can't use the virtual modems with COM1-4.

virtual COM port 16 bits changing.

- ATI3 now reply V1.8 instead of V1.6

- AT*i now reply the same thing as ATI3.

## TECH0264  13/11/98

New : firmwares WW.L55 (fixing analog + LAN problem).

New : SX456ETH.SYS v1.02

Fix hazardous problem : MAC address = "00 00 00 00 00 00".

Fix: Win98, when he launch the machine the network is not loaded. Once the machine

launched, he remove the card and insert it again, the network start to load.

Fixed : INF file GSM part, "Protocol RLP" message fixed.

Problem known

-------------

"HTML LAN documentation" FR/GE link missing. Patch file available: LANPATCH.EXE

HTML UK documentation link is ok.

## TECH0265  17/11/98 (OEM 18/11/98)

New: LOADER v4.05

ISDN driver version under Win9x : v1.8

ISDN driver version under WinNT4 : v1.7

TECH0264 and TECH0254: New PSTN firmwares:

Fixed:  Win95 DUN crashed. The firmware designed to support v.90 only, as well as the firmware to support k56flex only, will work just fine the first time you make a connection.  After that, the next time you attempt to use it the modem does not respond.  Unplug/plug the modem or reboot is necessary to repeat the cycle.

Fixed: "HTML LAN documentation" FR/GE link missing.

Fixed: In the properties of LAN adapter the name is now "SX456ETH" instead of "COM One MC220...".

Problem known

-------------

Under Win95/98 if you use the built in Novell NetWare clients:

Manufacturer / Client :

- Microsoft / "Client for Novell networks"

- Novell / "Novell NetWare ..."

You might encounter corruptions when copying big files (over 20 Mb) trough a NetWare network.

Fixed from LAN driver v1.20

### Patch files released

9901a.exe

PC Card ISDN under WinNT 4 : patch file

---------------------------------------

Apply to WinNT 4 ISDN driver v1.5 to v1.7

SX2CTRL.SYS  Version 1.03.00.00  01/1998

Fix: hazardous disconnections or no connection at all.

Symptom : PSTN trough ISDN work, but ISDN only don't work.

Copy it over the previous file into C:\WINNT\SYSTEM32\DRIVERS\ then

reboot the system.

Worked for: Compaq Armada 7400, Toshiba Tecra 8000.

9901b.exe  (obsolete: replaced by 9902a)

PC Card ISDN under Win95/98 : patch file 01/1999

------------------------------------------------

Apply to Win95/98 ISDN driver up to v1.8

Fix: shut down problems after the installation of CAPI function.

Copy these 2 files over the previous files into

C:\WINDOWS\SYSTEM\ then reboot the system.

Usual error message: fatal error "0 E" ("Une erreur fatale 0E est apparue").

BOOTLOG.TXT sample:

...

Terminate = User

Terminate = Query Drivers

EndTerminate = Query Drivers

Terminate = Unload Network

EndTerminate = Unload Network

Terminate = Reset Display

EndTerminate = Reset Display

EndTerminate = User

9902a.exe

PC Card ISDN under Win95/98 : patch file 02/1999

------------------------------------------------

Apply to ISDN Win95/98 driver v1.8 only.

Fix: shut down problems (hang or blue screen) after

the installation of CAPI function.

1. Right click onto the SX128 icon in the task bar,

then Quit SX128.

2. Copy these patch files over the previous files

into C:\WINDOWS\SYSTEM\

3. Reboot the system.

Notes:

- This file replace the patch 9901b.

- SX128 panel version will display "sx128 version 2.0"

9902b.zip

PC Card ISDN under WinNT 4 : patch file 02/1999

------------------------------------------------

Apply to driver v1.7 only.

Fix: SX128 Test reply "Local configuration not correct" error message.

Connection only works after removing ISDN cable from adapter then plug it back.

Explanation: This problem appear on ISDN bus activated (even if none B channel isn't used). Problem can appear behind some PBX and some countries (Switzerland, Germany), for example with an ISDN phone on the same bus.

Installing the fix:

1. Control panel / Services / Stop "SX128 Isdn Stack".

2. Copy SX2CARR.EXE v1.02 over previous file (v1.00) into C:\WINDOWS\SYSTEM32.

3. Reboot the system.

## TECH0330 (DISK0213)   13/03/99

New: Loader v4.09

ISDN driver version under Win9x : v2.0

ISDN driver version under WinNT4 : v1.7

Fixed: Include all previous patches.

Changed: Autorun function removed (after installation PC reboot, some users was thinking they need to install the drivers again).

Changed: The LAN installer copy MDM220.INF under the name of MDM220E.INF

Note:

- MDM220.INF and MDM220E.INF are the same files.

- The name of MDM220.INF on root of CD-Rom is normal.

Explanation: with Sony Vaio Prince (one PCMCIA slot, PCMCIA CD-Rom drive), during LAN installation the file MDM220.INF disappeared.

### Firmwares files included

Note: The Loader automatically check if MC220 contain a 2 Mb or 4 Mb flash memory. AT*i show "L" for 2 Mb firmwares and "M" for 4 Mb firmwares.

2 Mb firmwares files:

No GSM support:

File1=COM1WW.L61,K56Flex+V90 PSTN / ISDN

V90 (no K56 Flex):

com1vmot.L57,MOTOROLA STARTAC / SLIMLite

com1vmot.L57,MOTOROLA 8700 / 8400 / d460

com1verk.L58,ERICSSON series 3, 6, 7, 8

com1vnk2.L57,NOKIA 2110

com1vnok.L57,NOKIA 8110 / 3110 / 3810

com1vnk6.L58,NOKIA 6110 / 5110

com1vs10.L57,SIEMENS S10

com1vsie.L56,SIEMENS S4

com1vs10.L57,SONY CMD-X2000

com1vsie.L56,SONY CM-DX1000

com1vz1.L56,SONY Z1

K56 Flex (no V90):

com1mot.L57,MOTOROLA STARTAC / SLIMLite

com1mot.L57,MOTOROLA 8700 / 8400 / d460

com1erk.L58,ERICSSON series 3, 6, 7, 8

com1nk2.L57,NOKIA 2110

com1nok.L57,NOKIA 8110 / 3110 / 3810

com1nk6.L58,NOKIA 6110 / 5110

com1s10.L57,SIEMENS S10

com1sie.L57,SIEMENS S4

com1s10.L57,SONY CMD-X2000

com1sie.L57,SONY CM-DX1000

com1z1.L57,SONY Z1

4 Mb firmwares files:

No GSM support:

No "COM1WW.xxx" file anymore.

Important: If you want to update a 4 Mb card, you must upgrade with a GSM file even if you don't own a GSM phone.

GSM support (V90 + K56 Flex)

com1mot.M02,MOTOROLA STARTAC / SLIMLite

com1mot.M02,MOTOROLA 8700 / 8400 / d460

com1erk.M03,ERICSSON series 3, 6, 7, 8

com1nk2.M02,NOKIA 2110

com1nok.M02,NOKIA 8110 / 3110 / 3810

com1nk6.M03,NOKIA 6110 / 5110

com1s10.M02,SONY CMD-X2000

com1sie.M01,SONY CM-DX1000

com1z1.M01,SONY Z1

com1s10.M02,SIEMENS S10

com1sie.M01,SIEMENS S4

Problems known

--------------

Win98 SE (Second Edition): LAN don't work

-----------------------------------------

Symptom: MAC address empty.

Solution: update LAN driver to v1.20 or next.

We recommend to uninstall current drivers before to install the new one. Then upgrade the card again.

WinNT 4 with LAN installed : RAS PSTN connection problems

---------------------------------------------------------

Symptom: Error message 651 when trying to do a PSTN connection with RAS.

Modem work under HyperTerminal.

Work around: unplug the LAN adapter, do the PSTN connection, then replug the LAN adapter (LAN will still work).

Solution: Fixed with patch 9906b. Fixed with TECH release (TECH0359) We recommend to upgrade.

Note this anomaly does not occurs under Win9x.

WinNT 4 with ISDN installed : SX2TAPI ports disappeared

-------------------------------------------------------

Symptom: When trying to using "Remote Access Setup", Error message: "The following ports have been removed from the RAS configuration because they are no longer installed on the computer".

Network / Remote Access Service / Remote Access Setup / can't add "SX2TAPI" because they doesn't appear.

Workaround:

Network / Remote Access Service / Adapters, remove "SX128" reboot.

Add the adapter: "SX2TAPI - WAN Miniport Adapter"

Network / Remote Access Service / Remote Access Setup

Add the two RAS devices: "SX2TAPI"

This hazardous problem affect all previous ISDN WinNT 4 drivers (up to v1.7). The fix will be included into next TECH.

Fixed: from NT ISDN driver v1.71

Under WinNT 4 when installing ISDN : "Invalid command-line arguments"

---------------------------------------------------------------------

Symptom: When installing the ISDN option for the MC220, after adding the ports, NT goes into a process of reviewing bindings etc.

When it gets as far as "TCP/IP Protocol is reviewing bindings", this error error message in a dialog-box saying "SETUP / Invalid command-line arguments. Unable to continue."

We've no explanation currently to this problem.

Under Win98 when using ISDN : accessing disk drive hang up ISDN

---------------------------------------------------------------

Problem known with Toshiba Satellite 4010 under Win98. Bios problem.

Toshiba Satellite 4060 and 4080 have the same problem.

Under Win98 when installing ISDN

--------------------------------

Error message : "ISDNSEL : The ISDN stack is not installed on your system. Please install it before running this setup program."

This error message do not cause problems. Don't care about it.

Fixed from ISDN driver v2.02 (TECH0359).

Under WinNT 4 when installing ISDN

----------------------------------

- Error message "The PC card modem is not installed or is not plugged. Please install it, and plug the PC card before running again the setup program".

- French error message: "Installation : La carte PCMCIA n'est pas installée ou n'est pas insérée dans son emplacement. Veuillez installer la carte PCMCIA avant de relancer le programme d'installation".

This error message can appear if:

1. The PC Card modem is not installed at all (Control panel / Modems : empty). You must add a new modem and let Windows detect the card.

2. The PC Card was installed with a old MDM220.INF file (ex: from floppy disk "TECH0021", or CD TECH0265).

3. The PC Card modem is installed with the wrong driver (ie. "Standard Modem").

To solve points 1 and 3, you must remove the PC Card modem driver from NT (you may have to install a unreal modem in order to let RAS reconfigure), restart Windows, then add a new modem using the driver MDM220.INF from the CD-Rom provided with ISDN drivers.

LAN with Windows 98 SE

----------------------

Main symptom : No MAC Address.

We confirm this version of the LAN driver can't work under Win98 Second Edition.

Fixed from LAN driver v1.23 (included into next TECH release).

Installing LAN under WinNT 4

----------------------------

- English error message: "OEMSETUP.INF file missing".

- French error message: "Message du programme d'installation: Le programme d'installation ne peut pas trouver OEMSETUP.INF ou OEMSETNT.INF Veuillez entrer le nouveau chemin pour le fichier OEMSETUP.INF"

Occurs when trying to add a network adapter from "Control panel / Network".

- French error message "Erreur: La migration de WinSock 1.1 vers 2.0 a échoué (   0x2). Le bilan des liaisons et la configuration vont continuer, mais vos composants de gestion du réseau ne seront peut-être pas correctement configurés."

Solution:

This is not the right way to install MC220 LAN driver, you must install the card as a modem + dialup networking, then run the SETUP.EXE provided (select the LAN option, then upgrade the card for LAN with the Loader).

- To check if LAN driver is installed, look into the menu Start / COM One Platinium Card. If you see "LAN Manual" this mean LAN driver files are installed.

- To check if card is upgraded for LAN, under HyperTerminal type "at*i" the word "LAN" should appear.

Call Back with MSN and sub address

----------------------------------

Putting "#0556788382*123" into SX128 / "MSN / Sub address" does NOT work.

You can only set a MSN OR a sub address.

No change planed.

Error message in Event Viewer

-----------------------------

PSTN work, but ISDN don't.

Event log show: "The SX2API service depends on the SX2CTRL which failed to start because of the following error: The device is not ready".

We recommend to get in touch with COM One.

Missing file "SETUP.DLL"

------------------------

By default, system files are hidden by Windows 95/98. If you copy the CD to a hard disk, Windows will not copy system files.

### Patch files released

9904a

PC Card GSM under Win98 : patch file 9904a 04/1999

--------------------------------------------------

Apply to Windows 98 only.

Fix: error message "Modem not accessible".

Explanation: PC Card Energy saving switch off PC Cards. Applications that wish to use

the modem only let 1 sec before to sent commands.

Our PC Card with GSM feature requires 2 sec to be ready. This patch disable energy

saving function for PC Cards.

Installing the fix:

Double click the REG file.

9905a

PC Card LAN : patch file 05/1999 - SX456ETH.SYS v1.04

-----------------------------------------------------

Fix Siemens Scenic Mobile 350 under Win9x and WinNT 4.

Copy over the previous file:

- Windows 95/98: C:\WINDOWS\SYSTEM

- Windows NT 4 : C:\WINDOWS\SYSTEM32\DRIVERS\

Then take off the PC Card, wait about 5 seconds, then plug it again.

9905b

PC Card LAN : patch file 02/1999

--------------------------------

Specific patch for Siemens Scenic Mobile 510.

Copy over the previous file:

- Windows 95/98: C:\WINDOWS\SYSTEM

- Windows NT 4 : C:\WINDOWS\SYSTEM32

Then take off the PC Card, wait about 5 seconds, then plug it again.

Product version v1.01

9906a

PC Card LAN : patch file 06/1999 - SX456ETH.SYS v1.05

-----------------------------------------------------

20% Faster LAN speed.

This LAN driver work with all LAN firmwares.

Copy over the previous file:

- Windows 95/98: C:\WINDOWS\SYSTEM

- Windows NT 4 : C:\WINDOWS\SYSTEM32\DRIVERS\

Then take off the PC Card, wait about 5 seconds, then plug it again.

9906b

PC Card LAN : patch file 06/1999 - SX456ETH.SYS v1.20

-----------------------------------------------------

- Faster LAN speed (requires latest firmware (from L71 or M11)).

- Fix problems when using PSTN + LAN simultaneously under WinNT 4.

Copy over the previous file:

- Windows 95/98: C:\WINDOWS\SYSTEM

- Windows NT 4 : C:\WINDOWS\SYSTEM32\DRIVERS\

Then take off the PC Card, wait about 5 seconds, then plug it again.

## TECH0359 29/09/99

Downloadable from COM One web site since October 5th 1999.

OEM version : TECH0385 (12 Oct 99).

Versions included

LAN driver Win9x : SX456ETH.SYS v1.20

LAN driver WinNT4 : SX456ETH.SYS v1.26

ISDN driver Win9x : SX128 v2.02

ISDN driver WinNT 4 : SX128 v1.71

LOADER v4.10

LAN now work under Windows 98 SE.

LAN speed improved (about 30%).

GSM Firmwares Ericsson issues fixed ("no dialtone", "no carrier").

GSM German D1 network : fax issue fixed.

Note: from v1.20 the LAN driver only work if card is upgraded with the new firmware provided (from L70/M10 and nexts), use Loader to upgrade.

2 Mb firmwares files:

K56 Flex is unavailable if GSM or LAN is enabled. You can select V90 only.

No GSM support firmware:

File1=COM1WW.L71,K56Flex+V90 PSTN / ISDN

GSM + PSTN V90:

com1vmot.L71,MOTOROLA STARTAC / SLIMLite

com1vmot.L71,MOTOROLA 8700 / 8400 / d460

com1verk.L71,ERICSSON series 3, 6, 7, 8

com1veri.L73,ERICSSON i888

com1vnk2.L71,NOKIA 2110

com1vnok.L71,NOKIA 8110 / 3110 / 3810

com1vnk6.L71,NOKIA 6110 / 5110

com1vs10.L71,SIEMENS S10

com1vsie.L71,SIEMENS S4

com1vs10.L71,SONY CMD-X2000

com1vsie.L71,SONY CM-DX1000

com1vz1.L71,SONY Z1

4 Mb firmwares files:

No GSM support firmware:

No "COM1WW.xxx" file anymore. Loader use automatically Nokia 6110 firmware.

GSM support (V90 + K56 Flex)

com1mot.M11,MOTOROLA STARTAC / SLIMLite

com1mot.M11,MOTOROLA 8700 / 8400 / d460

com1erk.M11,ERICSSON series 3, 6, 7, 8

com1eri.M12,ERICSSON i888

com1nk2.M11,NOKIA 2110

com1nok.M11,NOKIA 8110 / 3110 / 3810

com1nk6.M11,NOKIA 6110 / 5110

com1s10.M11,SONY CMD-X2000

com1sie.M11,SONY CM-DX1000

com1z1.M11,SONY Z1

com1s10.M11,SIEMENS S10

com1sie.M11,SIEMENS S4

New

---

- WinNT 4 : ISDN installation improved, no need any more to add ISDN lines manually.

- Add Ericsson i888 (GSM Phone) support.

Problems fixed

--------------

- Symptom: After installing ISDN, if you already own LAN installed, then the LAN documentation was displayed instead of ISDN.

Changed: At the end of SETUP, the documentation displayed will be LAN if ISDN is installed, or ISDN if LAN is installed. Documentations priority: PSTN, GSM, ISDN, LAN.

- In PSTN mode, then GSM command AT+CBST=7 do not return "ERROR" any more.

- Symptom: Modems drivers "SX128 MODEM #2" (or #3 …).

Explanation: Modification done in the installation process. Now if you install ISDN driver over a previous ISDN driver, "SX128 MODEM #2" will not be added again.

With previous TECH you needed to install ISDN driver (Add/Remove program : "COMOne Platinium Card"), then install again.

- WinNT 4 with ISDN installed : SX2TAPI ports disappeared

Symptom: When trying to using "Remote Access Setup", Error message: "The following ports have been removed from the RAS configuration because they are no longer installed on the computer".

Network / Remote Access Service / Remote Access Setup / can't add "SX2TAPI" because they doesn't appear.

### Patch files released

0002a

Windows 95/98

"EnablePowerManagement" is activated back.

Card is now see by Windows as a multifunction card.

New phones in the Loader: Ericsson T18 and T28.

Windows NT 4

Add CAPI (16 and 32 bits).

# MC222

## TECH0390 13/12/99

CD for the PC Card Platinium Card MC222 (2 in 1 (GSM/ISDN)).

First drivers CD for MC222.

This CD is based on MC220 TECH0359.

Include drivers for : Windows 95, 98, NT 4.

Not compatible with Windows 2000.

New from TECH0359:

- Automatic installation of TAPI and TAPI Wan MiniPort.

- CAPI v2.0 (32 bits only) available under WinNT 4.

2 Mb firmwares files:

[4T129000000SXX.1]

T1=2in1 ISDN / GSM

T5=2in1 ISDN / GSM

Option=IG

H1=COM1_SA-MC222_CARD-7107

H5=COM1_SA-MC222_CARD-7107

File1=com1nk6.S10,No GSM

File2=com1eri.S10,ERICSSON i888, T28S

File3=com1erk.S10,ERICSSON series 6/7/8, T18S

File4=com1mot.S10,MOTOROLA STARTAC / SLIMLite

File5=com1mot.S10,MOTOROLA 8700 / 8400 / d460

File6=com1nk2.S10,NOKIA 2110

File7=com1nok.S10,NOKIA 8110 / 3110 / 3810

File8=com1nk6.S10,NOKIA 6110 / 5110

File9=com1s10.S10,SIEMENS S10

File10=com1sie.S10,SIEMENS S4

File11=com1s10.S10,SONY CMD-X2000

File12=com1sie.S10,SONY CM-DX1000

File13=com1z1.S10,SONY Z1

4 Mb firmwares files:

[4T129000000RXX.1]

T1=2in1 ISDN / GSM

T5=2in1 ISDN / GSM

Option=IG

H1=COM1_SA-MC222_CARD-7107

H5=COM1_SA-MC222_CARD-7107

File1=com1nk6.R10,No GSM

File2=com1eri.R10,ERICSSON i888, T28S

File3=com1erk.R10,ERICSSON series 6/7/8, T18S

File4=com1mot.R10,MOTOROLA STARTAC / SLIMLite

File5=com1mot.R10,MOTOROLA 8700 / 8400 / d460

File6=com1nk2.R10,NOKIA 2110

File7=com1nok.R10,NOKIA 8110 / 3110 / 3810

File8=com1nk6.R10,NOKIA 6110 / 5110

File9=com1s10.R10,SIEMENS S10

File10=com1sie.R10,SIEMENS S4

File11=com1s10.R10,SONY CMD-X2000

File12=com1sie.R10,SONY CM-DX1000

File13=com1z1.R10,SONY Z1

FAQ

---

Q: Why there is two INF file onto the CD-Rom? (MF222.INF and MDM222.INF)

A: This is because MC222 is declared as a Multifunction card into Windows.

Q: After installation of drivers, a new directory (C:\UNINSTAL.000) appeared.

A: It is a copy of MC222 drivers from the CD-Rom. You can delete it.

Known problem: CAPI's HDLC mode don't work.