MC220 (4 in 1) PC Card - Hardware schematic

v1.5 - DD / COM One - 07/1999

This is a basic schematic showing the links between the main components.

- P = PCMCIA connector (68 pins).

- C = Connector ISDN/GSM/LAN (15 pins).

- LI = PSTN line interface.

- CPLD = Complex programmable logical device (programmable hardware).

- MC Rockwell = Micro Controller Rockwell.

PCMCIA

Controller

MC

Rockwell

DSP Rockwell

LI

FLASH ROM (256 Kb)

P

RAM (128 Kb)

Dual port

memory

CPLD

C

## The firmware mainly modify these components :

Flash Rom, CPLD, MC Rockwell, and DSP.

## The GSM/ISDN/LAN functions are mainly done by these components :

CPLD for programmable digital processing.

Dual port memory is used for high speed transfers (firmware upgrade, ISDN, LAN).

## Hardware clocks :

Micro controller Rockwell : 28 MHz.

DSP Rockwell : 56 MHz.

# Firmwares PC Card COM One Platinium MC220 4 in 1

# Firmwares CIS

Firmwares PC Card COM One Platinium MC220 4 in 1

CIS = Card Information Structure

| CIS | MDM220.INF date | Firmware version |
| --- | --- | --- |
| 8507 | 04/1998 | 0 - 29 |
| 2F9A | 05/1998 | 30 - 49 |
| 43F1 | 07/1998 | >49 LAN not activated |
| 32CA | 09/1998 | >49 LAN activated |

# AT*i details

### AT*I reply :

“ Product ID ” <CR><LF>

“ Firmware type ” <CR><LF>

“ Modulation MAX ” <CR><LF>

[“ GSM phone ” <CR><LF>]

[“ Network ” <CR><LF>]

“ Firmware version ”

“ product ID ” :

PCCOM		carte PCMCIA MC220

“ Type firmware ” :

PSTN

DUO		(PSTN + ISDN)

TRIO		(PSTN + ISDN + GSM or LAN)

QUAT		(PSTN + ISDN + GSM + LAN)

“ Modulation MAX ” :

V34+

K56Flex

V90

K56Flex+V90

“ GSM phone ”

NOKIA 2110

NOKIA 8110

NOKIA 6110

MOTOROLA

ERICSSON

SIEMENS S3/S4

SIEMENS S10

SONY Z1

“ Network ”

LAN  (appears only with TRIO firmware (no GSM support)).

“ Firmware version ”

vxx		xx = 00 to 99, and v = A to Z

# Main firmwares evolutions

First DUO firmwares shipped: WW.L04 and L16

L01		K56 Flex.

L04		V90 support added.

L10		ISDN support added.

L50		LAN support added.

L70  / M10	LAN v1.20 support added.