Drivers releases for Light Modem V90 56K USB

For technical support

V1.12 - DD / COM One - 11/1999

# Drivers versions

V2.20

First version shipped.

V2.32

- Connection speed increased in V90 mode (about 2 to 4 Kb/s).

- Previous driver set by default LM56K to COM2, this can cause problems with Toshiba notebooks (COM2 seems to not work on these computers). Now the driver use COM3 by default.

- Fixed (used in France only) : Windows 98 plantait (écran bleu: Erreur fatale) lors d'un déconnexion en mode Minitel dans BVRP WinPhone v2.10a.

V2.50

- Fixed : connection work in Germany.

- Problem known : modem connect corrrectly to a provider supporting 33600 max. But if you force 33600 (AT+MS=34) and try to connect to a V90 provider, handshaking fail (NO CARRIER).

# TECH0302  CD-Rom  February 1999

Driver version v2.20

Paper documentation remark : documentation recommend CPU 166 MMX, box says 200 MMX. We recommend 200 MMX.

# TECH0367 CD-Rom  August 1999

- Driver version v2.50

- HTML documentation updated.

As the previous drivers, this one only work under Windows 98.

# Paper addendum DOCU0527 - 08/07/99

Avertissement

Actuellement, le Light Modem 56K n'est pas compatible avec Windows 2000. Toutefois, cette compatibilité est cours d'étude. Consultez notre site Web http://www.com1.fr pour vous tenir informé.

Warning

Currently, Light Modem 56K is not compatible with Windows 2000. However, this compatibility is in progress. Please, consult our web site http://www.com1.fr for more information.

# Troubleshooting

## Disabling Caller ID

LM56K USB, driver v2.50

Disabling Caller ID

DD / COM One 19 Oct 1999

Trouble solved

--------------

Solve the problem where the application don't get the second incoming call.

- Microsoft Windows 98 DUN server

- Symantec WinFax Pro v8

- Symantec PC Anywhere v8

- Most of other TAPI softwares.

Explanation

-----------

When LM56K is installed, Caller ID (modem see the number of caller) is activated.

In most of countries, to have Caller ID activated you must get an additional subscription.

If Caller ID is not activated on the phone line, LM56K will have troubles to answer incoming calls.

Note

----

Adding the string "AT #CID=0" in Control Panel / modems / additional string is not 100% efficient (Win98 sent this command then the AT#CID=1 from the INF).

Modification to do

------------------

1. In the directory:

c:\windows\other\inf

2. Edit the file:

mdmslusb.inf

3. Search for the string "#cid"

4. Here is the line you will find:

HKR, EnableCallerID,           1,, "at#cid=1<cr>"

5. Replace "#cid=1" by "#cid=0".

6. In the directory:

c:\windows\other\inf

7. Delete the files:

Drvdata.bin and Drvidx.bin

8. Open "My computer" / "Control panel" / "Modems"

Delete: "COMOne Light Modem 56K"

9. Restart the computer.

Windows will redetect the modem and find automatically the driver.